7th Lab – Three Way Traffic Light Control Using AT89C51 Microcontroller

Implementation:

Keil:

Open Keil µVision → Go to Project → New µVision Project → Set a project name and save it → In the Select Device for Target window, search and select AT89C51 → Click OK → When prompted, click No → In Project window, right-click Source Group 1 under Target 1 → Select Add New Item to Group 'Source Group 1' → Create and save a new C file → Write the code below and save:

Source Code:

#include <reg51.h>

// Traffic Light 1 (P2.0 - P2.2)

sbit RED_1    = P2^0;

sbit YELLOW_1 = P2^1;

sbit GREEN_1  = P2^2;

// Traffic Light 2 (P2.3 - P2.5)

sbit RED_2    = P2^3;

sbit YELLOW_2 = P2^4;

sbit GREEN_2  = P2^5;

// Traffic Light 3 (P3.0 - P3.2)

sbit RED_3    = P3^0;

sbit YELLOW_3 = P3^1;

sbit GREEN_3  = P3^2;

void delay(unsigned int t)

{

unsigned int i, j;

for(i = 0; i < t; i++)

for(j = 0; j < 1275; j++);

}

// Phase 1: 1 - RED, 2 - YELLOW, 3 - GREEN

void signal_phase1()

{

RED_1 = 1; YELLOW_1 = 0; GREEN_1 = 0;

RED_2 = 0; YELLOW_2 = 1; GREEN_2 = 0;

RED_3 = 0; YELLOW_3 = 0; GREEN_3 = 1;

delay(200);

}

// Phase 2: 1 - GREEN, 2 - RED, 3 - YELLOW

void signal_phase2()

{

RED_1 = 0; YELLOW_1 = 0; GREEN_1 = 1;

RED_2 = 1; YELLOW_2 = 0; GREEN_2 = 0;

RED_3 = 0; YELLOW_3 = 1; GREEN_3 = 0;

delay(200);

}

// Phase 3: 1 - YELLOW, 2 - GREEN, 3 - RED

void signal_phase3()

{

RED_1 = 0; YELLOW_1 = 1; GREEN_1 = 0;

RED_2 = 0; YELLOW_2 = 0; GREEN_2 = 1;

RED_3 = 1; YELLOW_3 = 0; GREEN_3 = 0;

delay(200);

}

// Sequence control

void traffic_light_sequence()

{

signal_phase1();

signal_phase2();

signal_phase3();

}

void main()

{

P1 = 0x00;

P2 = 0x00;

P3 = 0x00;

while(1)

{

traffic_light_sequence();

}

}

→ Right-click Source Group 1 → Add Existing Files to Group → Select the saved .c file → Click Add and Close → Right-click Target 1 → Options for Target → Go to Output tab → Check "Create Hex File" → Go to Target tab → Set Xtal (MHz) to 11.0592 → Click OK → Press F7 to build the project → If there are no errors or warnings, your code is okay.

Proteus:

Open Proteus → Click New Project → Set project name → Keep clicking Next until Finish → Click Finish → From the left sidebar, select Component Mode → Click P → Search and add AT89C51 → Add Traffic Lights

Component Placement:

Place U1 (AT89C51) on the workspace → Place Traffic Lights on three sides (each with 3 LEDs: Red, Yellow, Green)

Wiring:

- Traffic Light 1:

  - P2.0 → Red

  - P2.1 → Yellow

  - P2.2 → Green

- Traffic Light 2:

  - P2.3 → Red

  - P2.4 → Yellow

  - P2.5 → Green

- Traffic Light 3:

  - P3.0 → Red

  - P3.1 → Yellow

  - P3.2 → Green

Programming:

Double-click U1 → Set clock frequency = 11.0592MHz → Browse and select the U1 HEX file generated from Keil → Click OK

Simulation:

Click the Play button → If Traffic Lights blink correctly in the 3-way sequence (RED – YELLOW – GREEN cycling between directions), your setup is working → Save the project

Diagram: