6th Lab – RIP based FLSM Network Topology

IPV4 Subnetting Concept

Rules:

1. Identify network & host requirements

2. Satisfy Network & host requirements

3. Find out Block size

4. Find out network address

5. Find out broadcast address

6. Find out valid host ip address range

7. Find out custom subnet mask & CIDR

Requirements:

IT LAN (60)

MIS LAN (60)

Sales LAN (60)

Class C:

N     N    N   H

192.168.100.0/24

FLSM - Fixed length subnet mask

VLSM - Variable length subnet mask

Network: 2^2 =4

Host: 2^6-2 = 62

Block size: 2^6 = 64

IP Address Allocation Table:

| Subnet No. | Network Address(N.A) | Broadcast Address(B.A) | Valid Host IP (Address Range) |
| --- | --- | --- | --- |
| 1st | 192.168.100.0 | 192.168.100.63 | 192.168.100.1 - 192.168.100.62 |
| 2nd | 192.168.100.64 | 192.168.100.127 | 192.168.100.65 - 192.168.100.126 |
| 3rd | 192.168.100.128 | 192.168.100.191 | 192.168.100.129 - 192.168.100.190 |
| 4th | 192.168.100.192 | 192.168.100.255 | 192.168.100.193 - 192.168.100.254 |

Custom Subnet Mask: 255.255.255.192

CIDR: /26

Task:

Subnet the following:

192.168.17.0/24

Network: 5

Host: 30

- Create IP allocation table

- Configure Topology

Topology Example:

- IP allocation Table

Network: 5

Host: 30

Address: 192.168.17.0/24

- Solution:

Network: 23 = 8

Host: 25-2 = 30

Block Size: 25 = 32

| Subnet No. | Network Address (N.A) | Broadcast Address (B.A) | Valid Host IP (Address Range) |
| --- | --- | --- | --- |
| 1st | 192.168.17.0 | 192.168.17.31 | 192.168.17.01 - 192.168.17.30 |
| 2nd | 192.168.17.32 | 192.168.17.63 | 192.168.17.33 - 192.168.17.62 |
| 3rd | 192.168.17.64 | 192.168.17.95 | 192.168.17.65 - 192.168.17.94 |
| 4th | 192.168.17.96 | 192.168.17.127 | 192.168.17.97 - 192.168.17.126 |
| 5th | 192.168.17.128 | 192.168.17.159 | 192.168.17.129 - 192.168.17.158 |
| 6th | 192.168.17.160 | 192.168.17.191 | 192.168.17.161 - 192.168.17.190 |
| 7th | 192.168.17.192 | 192.168.17.223 | 192.168.17.193 - 192.168.17.222 |
| 8th | 192.168.17.224 | 192.168.17.255 | 192.168.17.225 - 192.168.17.254 |

Table:

Custom Subnet Mask: 255.255.255.224

CIDR: /27

Topology

- Network Summary

| Switch Name | Subnet | Connected PCs |
| --- | --- | --- |
| S0 (IT) | 192.168.17.32/27 | PC0, PC1 |
| S1 (MIS) | 192.168.17.64/27 | PC2, PC3 |
| S2 (Sales) | 192.168.17.160/27 | PC4, PC5 |
| Serial Link | 192.168.17.192/27 | R1 ↔ R2 |

- Router Configuration (CLI)

Router R1

Router> enable

Router# configure terminal

R1(config)# hostname R1

R1(config)# interface fastethernet 0/0

R1(config-if)# ip address 192.168.17.33 255.255.255.224

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# ip dhcp pool IT

R1(dhcp-config)# network 192.168.17.32 255.255.255.224

R1(dhcp-config)# default-router 192.168.17.33

R1(dhcp-config)# exit

R1(config)# interface fastethernet 0/1

R1(config-if)# ip address 192.168.17.65 255.255.255.224

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# ip dhcp pool MIS

R1(dhcp-config)# network 192.168.17.64 255.255.255.224

R1(dhcp-config)# default-router 192.168.17.65

R1(dhcp-config)# exit

R1(config)# interface serial 0/0/0

R1(config-if)# ip address 192.168.17.193 255.255.255.224

R1(config-if)# clock rate 148000

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# router rip

R1(config-router)# network 192.168.17.32

R1(config-router)# network 192.168.17.64

R1(config-router)# network 192.168.17.192

R1(config-router)# no auto-summary

R1(config)# Ctrl+Z

R1# write

Router R2

Router> enable

Router# configure terminal

R2(config)# hostname R2

R2(config)# interface fastethernet 0/0

R2(config-if)# ip address 192.168.17.161 255.255.255.224

R2(config-if)# no shutdown

R2(config-if)# exit

!--- DHCP Configuration ---

R2(config)# ip dhcp pool Sales

R2(dhcp-config)# network 192.168.17.160 255.255.255.224

R2(dhcp-config)# default-router 192.168.17.161

R2(dhcp-config)# exit

R2(config)# interface serial 0/0/1

R2(config-if)# ip address 192.168.17.194 255.255.255.224

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config)# router rip

R2(config-router)# network 192.168.17.160

R2(config-router)# network 192.168.17.192

R2(config-router)# no auto-summary

R2(config)# Ctrl+Z

R2# write

- PC Configuration (PC0–PC5)

- Click on each PC (e.g., PC0).

- Go to Desktop tab > IP Configuration.

- Select DHCP.

- The PC will receive:

- IP Address

- Subnet Mask

- Default Gateway (from router)