7th Lab – RIP based FLSM Network (Large Subnet Design)

Task:

Subnet the following:

172.19.0.0/16

Network: 10

Host: 20

- Create IP allocation table

- Configure Topology

Topology Example

- IP allocation Table

Network: 10

Host: 20

Address: 172.19.0.0/16

- Solution:

Network: 24 = 16

Host: 212-2 = 4094

Block Size: 24 = 16

Table:

| Subnet No. | Network Address (N.A) | Broadcast Address (B.A) | Valid Host IP (Address Range) |
| --- | --- | --- | --- |
| 1st | 172.19.0.0 | 172.19.15.255 | 172.19.0.01 - 172.19.15.254 |
| 2nd | 172.19.16.0 | 172.19.31.255 | 172.19.16.01 - 172.19.31.254 |
| 3rd | 172.19.32.0 | 172.19.47.255 | 172.19.32.01 - 172.19.47.254 |
| 4th | 172.19.48.0 | 172.19.63.255 | 172.19.48.01 - 172.19.63.254 |
| 5th | 172.19.64.0 | 172.19.79.255 | 172.19.64.01 - 172.19.79.254 |
| 6th | 172.19.80.0 | 172.19.95.255 | 172.19.80.01 - 172.19.95.254 |
| 7th | 172.19.96.0 | 172.19.111.255 | 172.19.96.01 - 172.19.111.254 |
| 8th | 172.19.112.0 | 172.19.127.255 | 172.19.112.01 - 172.19.127.254 |
| 9th | 172.19.128.0 | 172.19.143.255 | 172.19.128.01 - 172.19.143.254 |
| 10th | 172.19.144.0 | 172.19.159.255 | 172.19.144.01 - 172.19.159.254 |
| 11th | 172.19.160.0 | 172.19.175.255 | 172.19.160.01 - 172.19.175.254 |
| 12th | 172.19.176.0 | 172.19.191.255 | 172.19.176.01 - 172.19.191.254 |
| 13th | 172.19.192.0 | 172.19.207.255 | 172.19.192.01 - 172.19.207.254 |
| 14th | 172.19.208.0 | 172.19.223.255 | 172.19.208.01 - 172.19.223.254 |
| 15th | 172.19.224.0 | 172.19.239.255 | 172.19.224.01 - 172.19.239.254 |
| 16th | 172.19.240.0 | 172.19.255.255 | 172.19.240.01 - 172.19.255.254 |

Custom Subnet Mask: 255.255.240.0

CIDR: /20

Topology

- Network Summary

| Switch Name | Subnet | Connected PCs |
| --- | --- | --- |
| Switch0 (IT) | 172.19.0.0/20 | PC0, PC1 |
| Switch1 (Server) | 172.19.16.0/20 | PC2, PC3 |
| Switch2 (Sales) | 172.19.144.0/20 | PC4, PC5 |
| Serial Link | 172.19.176.0/20 | R1 ↔ R2 |

- Router Configuration

Router R1

Router> enable

Router# configure terminal

Router(config)# hostname R1

R1(config)# interface fastethernet 0/0

R1(config-if)# ip address 172.19.0.1 255.255.240.0

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# ip dhcp pool IT

R1(dhcp-config)# network 172.19.0.0 255.255.240.0

R1(dhcp-config)# default-router 172.19.0.1

R1(dhcp-config)# exit

R1(config)# interface fastethernet 0/1

R1(config-if)# ip address 172.19.16.1 255.255.240.0

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# ip dhcp pool Server

R1(dhcp-config)# network 172.19.16.0 255.255.240.0

R1(dhcp-config)# default-router 172.19.16.1

R1(dhcp-config)# exit

R1(config)# interface serial 0/0/0

R1(config-if)# ip address 172.19.176.1 255.255.240.0

R1(config-if)# clock rate 148000

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# router rip

R1(config-router)# network 172.19.0.0

R1(config-router)# network 172.19.16.0

R1(config-router)# network 172.19.176.0

R1(config-router)# no auto-summary

R1(config)# Ctrl+Z

R1# write

Router R2

Router> enable

Router# configure terminal

Router(config)# hostname R2

R2(config)# interface fastethernet 0/0

R2(config-if)# ip address 172.19.144.1 255.255.240.0

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config)# ip dhcp pool Sales

R2(dhcp-config)# network 172.19.144.0 255.255.240.0

R2(dhcp-config)# default-router 172.19.144.1

R2(dhcp-config)# exit

R2(config)# interface serial 0/0/1

R2(config-if)# ip address 172.19.176.2 255.255.240.0

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config)# router rip

R2(config-router)# network 172.19.144.0

R1(config-router)# network 172.19.176.0

R2(config-router)# exit

R2(config)# Ctrl+Z

R2# write

- PC Configuration (PC0–PC5)

- Click on each PC (e.g., PC0).

- Go to Desktop tab > IP Configuration.

- Select DHCP.

- The PC will receive:

- IP Address

- Subnet Mask

- Default Gateway (from router)