8th Lab – RIP based VLSM Network Topology

Topology

- Network Summary

| Network Name | Subnet | Description |
| --- | --- | --- |
| Server-5 | 172.19.4.160/29 | Server0–Server2 |
| IT-17 | 172.19.4.128/27 | PC0–PC1 |
| MIS-100 | 172.19.4.0/25 | PC2–PC3 |
| Sales-1000 | 172.19.0.0/22 | PC4–PC9 |
| Serial Link | 172.19.4.168/30 | R1 ↔ R2 |

- Router Configuration

Router R1

Router> enable

Router# configure terminal

Router(config)# hostname R1

R1(config)# interface fastethernet 0/0

R1(config-if)# ip address 172.19.4.161 255.255.255.248

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# interface fastethernet 0/1

R1(config-if)# ip address 172.19.4.129 255.255.255.224

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# ip dhcp pool IT

R1(dhcp-config)# network 172.19.4.128 255.255.255.224

R1(dhcp-config)# default-router 172.19.4.129

R1(dhcp-config)# exit

R1(config)# interface serial 0/0/0

R1(config-if)# ip address 172.19.4.169 255.255.255.252

R1(config-if)# clock rate 148000

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# router rip

R1(config-router)# version 2

R1(config-router)# network 172.19.4.160

R1(config-router)# network 172.19.4.128

R1(config-router)# network 172.19.4.168

R1(config-router)# no auto-summary

R1(config)# Ctrl+Z

R1# write

Router R2

Router> enable

Router# configure terminal

Router(config)# hostname R2

R2(config)# interface fastethernet 0/0

R2(config-if)# ip address 172.19.4.1 255.255.255.128

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config)# ip dhcp pool MIS

R2(dhcp-config)# network 172.19.4.0 255.255.255.128

R2(dhcp-config)# default-router 172.19.4.1

R2(dhcp-config)# exit

R2(config)# interface fastethernet 0/1

R2(config-if)# ip address 172.19.0.1 255.255.252.0

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config)# ip dhcp pool Sales

R2(dhcp-config)# network 172.19.0.0 255.255.252.0

R2(dhcp-config)# default-router 172.19.0.1

R2(dhcp-config)# exit

R2(config)# interface serial 0/0/1

R2(config-if)# ip address 172.19.4.170 255.255.255.252

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config)# router rip

R2(config-router)# version 2

R2(config-router)# network 172.19.4.0

R2(config-router)# network 172.19.0.0

R2(config-router)# network 172.19.4.168

R2(config-router)# no auto-summary

R2(config)# Ctrl+Z

R2# write

- Configuration Server and PC (PC0–PC9):

In Static: Server0, Server1, Server2 (Server-5 Network)

- Subnet: 172.19.4.160/29

- IPs: 172.19.4.162, .163, .164

- Subnet Mask: 255.255.255.248

- Gateway: 172.19.4.161

Click on each PC (e.g., PC0).

- Go to Desktop tab > IP Configuration.

- Select DHCP.

- The PC will receive:

- IP Address

- Subnet Mask

- Default Gateway (from router)